NAME: GAYATHRI.P

REG NO: 192424324

CODE: ITA0329

COURSE: MOBILE COMPUTING

1) Given

Total bandwidth = 45 MHz = 45,000 kHz, Simplex channel bandwidth = 25 kHz

Full-duplex channels = Total bandwidth ÷ (2 × Simplex bandwidth)

= 45,000 ÷ (2 × 25)

= 45,000 ÷ 50

Answer: 900  channels

2) Given:

- Total voice channels = 560, Cluster size = 7 cells

Channels per cell = Total channels ÷ Cluster size

= 560 ÷ 7

Answer: 80 channels per cell

3) Given:

- Number of carriers = 100, Time slots per carrier = 8

Total voice channels = Carriers × Time slots

= 100 × 8

Answer: 800  channels

4) Given:

- Base station power = 80 W, Handset power = 4 W

P=10log(p1/p2)

= 10log(80/4)

=10log(20)

ANSWER:13.01db

5) Given:

- Carrier power = −60 dBm, Interference power = −92 dBm

C/I = Carrier power − Interference power

= −60 − (−92)

= −60 + 92

Answer: 32 dB

6)Given:

- Total spectrum = 42 MHz = 42,000 kHz, Bandwidth per channel = 300 kHz, Cluster size = 7 cells

Channels per cell = Total channels ÷ Cluster size

Total channels = 42,000 ÷ 300 = 140

Channels per cell = 140 ÷ 7

Answer: 20 channels per cell

7) Given:

- Initial received power = 64 W, Distance factor = 2, Path-loss exponent = 4

P2=p1/d^n

= 64 ÷ (2^4)

= 64 ÷ 16

Answer: 4 W

8) Given:

- Initial radius = (R), New radius = (2R), Path-loss exponent = 2

Power factor = Distance factor²

= (2^2=4)

Answer: 4 times the original power

9) Given:

- Cluster size = 4 cells, Channels per cell = 15

Total channels = Cluster size × Channels per cell

= 4 × 15

Answer: 60 channels

10) Given:

- Velocity = 100 km/h = 27.78 m/s, Carrier frequency = 900 MHz, Speed of light = \(3\times10^8\) m/s

Fd=vf/c

= (27.78 × 900 × 10⁶) ÷ (3 × 10⁸)

Answer: Approximately 83.3 Hz

11) Given:

- Frame duration = 5 ms, Number of slots = 8

Slot duration = Frame duration ÷ Number of slots

= 5 ÷ 8

Answer: 0.625 ms

12) Given:

- Total bandwidth = 24 MHz = 24,000 kHz, Carrier spacing = 200 kHz

Number of carriers = Total bandwidth ÷ Carrier spacing

= 24,000 ÷ 200

Answer: 120 carriers

13) Given:

- Transmitted power = 8 W = 8,000 mW

- Path loss = 33 dB

P=10log(p)

Transmitted power = (10log{(8000)=39.03) dBm

Received power = 39.03 − 33

Answer: 6.03 dBm

14) Given:

- Frequency = 1900 MHz, Distance = 2.5 km, Reference distance = 1 m

FSPL=32.44+20log10f+20log10d

= 32.44 + 20 log₁₀(1900) + 20 log₁₀(2.5)

Answer: Approximately 105.97 dB

15) Given:

- Number of cells = 6

- Calls per cell = 250

Total capacity = Number of cells × Calls per cell

= 6 × 250

Answer: 1,500 simultaneous calls

16) Given

- Call requests = 4,000 per second, Average call duration = 3 minutes = 180 seconds

Active calls = Calls per second × Duration in seconds

= 4,000 × 180

Answer: 720,000 active calls

17) Given

- Number of channels = 150, Bandwidth per channel = 25 kHz

Total bandwidth = Channels × Bandwidth per channel

= 150 × 25 = 3,750 kHz

Answer: 3.75 MHz

18)Given

- Number of frequency bands = 30, Guard band per gap = 40 kHz

Total guard bandwidth = (Number of bands − 1) × Guard bandwidth

= (30 − 1) × 40

= 29 × 40

Answer: 1,160 kHz = 1.16 MHz

19)Given

- Speed = 90 km/h, Cell radius = 1.5 km, Cell diameter = 2 × radius = 3 km

Handoffs = Distance travelled ÷ Cell diameter

= 90 ÷ 3

Answer: Approx 30 handoffs per hour

20)Given

- Regional centres = 8, Base stations per centre = 40, Users per base station = 250

Total users = Centres × Base stations × Users

= 8 × 40 × 250

Answer: 80,000 users

21)Given

- Total area = 400 km, Cell radius = 400 m = 0.4 km, Approximate cell area = \(R^2\)

Number of cells = Total area ÷ Cell area

Cell area = 0.4² = 0.16 km

Number of cells = 400 ÷ 0.16

Answer: 2,500 cells

22)Given

- Total spectrum = 6 MHz = 6,000 kHz, Bandwidth per channel = 30 kHz

Number of channels = Total bandwidth ÷ Channel bandwidth

= 6,000 ÷ 30

Answer: 200 channels

23.)Given

- Subscribers = 1,200, Calls per subscriber per hour = 4, Average call duration = 2.5 minutes

A=N×C×T60

= (1,200 × 4 × 2.5) ÷ 60

Answer: 200 Erlangs

24)Given

- Total bandwidth = 40 MHz, Bandwidth per user = 400 kHz

Users = Total bandwidth ÷ Bandwidth per user

= 40,000 ÷ 400

Answer: 100 users

25)Given:

- Number of base stations = 1,200, Users per base station = 600

Total users = Base stations × Users per station

= 1,200 × 600

Answer: 720,000 users